Guide for Firmware updating

Attention！

a.The drone’s firmware version V105 only works when with the V1.0.7 firmware controller and the programming software - LiteBeego v0.95

b. The drone’s firmware version V108 only works when with the V1.1.0 firmware controller and the programming software - LiteBeego v1.1.3

1.Introduction

This file aims to tell how to update the firmware of the drone.

1.1Install the driver

Take the battery out from the drone, press the reset button (as the picture shows) with a needle, hold until the drone connected with the computer through a USB wire, and its light on.

Right-click “This PC” and then follow the path “manage” - “Device Manager” - “Other devices” to find the “Unknown device”

Right-click the “Unknown device”, - choose “Update driver”, - choose “browse my computer for driver software” - find the downloaded file, follow the path “LiteBee Wing update/Firmware update/Driver software/” - choose “usbdriver” - click “Next” to update. If update successfully, this device(USB device (VID_***_PID_***)) will be shown in the file of “Universal Serial Bus controllers”. After the updating, disconnect the drone and computer.

1.2Update the firmware

Open the downloaded file, follow the path “LiteBee Wing update/Firmware update/Driver software/PhoenixUSBPro” to start the burning software. Click the “Image” to choose the firmware you want to update(path: LiteBee Wing update/Firmware update/Firmware/)

Press the reset button of the drone and hold until the drone connects with the computer and the “√” mark of the software became “×”. Release the button, and the update is under process automatically, when the process bar becomes 100%, the updating finished.